WareHouse Management System Using LORA-ESP32

Sender Door LORA Devices :

RFM95 Module connected with esp32 is use. This device will send

the “>DOOR_Num<” to the Receiver LORA Module every 10 seconds

Check Properly the Door Number and its respective gps location in csv file and fix it there.

Improper configuration leads to wrong Information on WebMap

Arduino Code :

#include <SPI.h>

#include <LoRa.h>

// Define the pins used by the LoRa transceiver module

#define ss   5   // Slave Select (SS) pin

#define rst 14   // Reset (RST) pin

#define dio0 2   // Digital Input/Output 0 (DIO0) pin

int counter = 0;

void setup() {

// Initialize Serial Monitor for debugging

Serial.begin(115200);

while (!Serial); // Wait for Serial Monitor to open

Serial.println("LoRa Sender");

// Setup LoRa transceiver module using specified pins for SS, RST, and DIO0

LoRa.setPins(ss, rst, dio0);

// Initialize the LoRa module with a specified frequency (915 MHz in this case)

// Keep trying until LoRa.begin() returns true, indicating successful initialization

while (!LoRa.begin(915E6)) {

Serial.println("."); // Display dots while waiting for initialization

delay(500);

}

// Change the sync word to 0xF3 to match the receiver's sync word

// This ensures that only LoRa messages with the same sync word are received

LoRa.setSyncWord(0xF3);

Serial.println("LoRa Initializing OK!");

}

void loop() {

// Display the packet number being sent in the Serial Monitor

Serial.print("Sending packet: ");

Serial.println(counter);

// Send a LoRa packet to the receiver

LoRa.beginPacket();

LoRa.print(">DOOR_3<"); // Transmit a message ('>DOOR_3<') to the receiver

LoRa.endPacket();

// Increment the counter for the next packet

counter++;

// Wait for a delay of 10 seconds before sending the next packet

delay(10000);

}

Receiver Door LORA Device :

Arduino Code :

#include <SPI.h>

#include <LoRa.h>

// Define the pins used by the LoRa transceiver module

#define ss   5  // Slave Select pin for LoRa

#define rst  14   // Reset pin for LoRa

#define dio0 2  // Digital IO pin for LoRa interrupt

// Define serial ports for communication

#define SerialMon Serial    // Serial monitor

#define SerialAT Serial1    // Serial communication with GSM modem

// GSM settings

char apn[] = "mnet";    // Access Point Name for GSM

char user[] = "";       // Username for GSM, if needed

char pass[] = "";       // Password for GSM, if needed

#define GSM_PIN ""      // PIN for SIM card, if required

#define TINY_GSM_MODEM_SIM7600  // GSM modem type

#define TINY_GSM_RX_BUFFER 1024  // Set RX buffer size for GSM communication

#include <TinyGsmClient.h>   // Library for GSM communication

#include "utilities.h"      // Utility functions

// Create an instance of the TinyGsmClient for GSM communication

TinyGsm modem(SerialAT);

// Define LED pins

#define red   12   // Red LED pin

#define blue  15   // Blue LED pin

#define green 13   // Green LED pin (Start LED)

// Server and path details for HTTP request

const char server[] = "ernd.tech";                  // Server address

const char path[] = "/test/csvmap.php?doorNumbers="; // Path including query parameters

// GSM client for HTTP

TinyGsmClient client(modem);

unsigned long startTime;

unsigned long duration = 60000;  // 1 minute in milliseconds

String receivedDoors = "";

void setup() {

// Initialize Serial Monitor for debugging

Serial.begin(115200);

while (!Serial);  // Wait until Serial Monitor is ready

Serial.println("LoRa Receiver");

// Setup LoRa transceiver module

LoRa.setPins(ss, rst, dio0);

// Start AT mode for GSM communication

SerialAT.begin(115200, SERIAL_8N1, MODEM_RX, MODEM_TX);

startModem();  // Initialize the GSM modem

// Start LoRa module on the specified frequency

while (!LoRa.begin(915E6)) {

Serial.println(".");

delay(500);

}

LoRa.setSyncWord(0xF3);  // Set LoRa sync word

Serial.println("LoRa Initializing OK!");

// Connect to GPRS for GSM communication

if (!modem.gprsConnect(apn, user, pass)) {

Serial.println("Failed to connect to GPRS");

return;

}

Serial.println("Connected to GPRS");

blinkLED(blue);  // Blink the blue LED to indicate connection

blinkLED(blue);

blinkLED(blue);

startTime = millis();  // Initialize start time

}

void loop() {

// Check for incoming LoRa packets for 1 minute

while (millis() - startTime < duration) {

if (LoRa.parsePacket()) {

String packet = "";

while (LoRa.available()) {

packet += (char)LoRa.read();

}

// Extract door number from the packet and add it to the list

int startPos = packet.indexOf(">") + 6;

int endPos = packet.indexOf("<");

String doorNum = packet.substring(startPos, endPos);

receivedDoors += doorNum + ",";

}

}

// Perform HTTP POST request with collected door numbers

if (receivedDoors.length() > 0) {

receivedDoors.remove(receivedDoors.length() - 1); // Remove last comma

postReq(receivedDoors);  // Send the collected data

}

// Reset variables for the next iteration

receivedDoors = "";

startTime = millis();

}

// Function to initialize the GSM modem

void startModem() {

// Power on and initialize GSM modem

pinMode(MODEM_PWRKEY, OUTPUT);

digitalWrite(MODEM_PWRKEY, HIGH);

delay(300);

digitalWrite(MODEM_PWRKEY, LOW);

pinMode(MODEM_FLIGHT, OUTPUT);

digitalWrite(MODEM_FLIGHT, HIGH);

// Attempt to start the modem

for (int i = 0; i < 3; ++i) {

while (!modem.testAT(5000)) {

Serial.println("Try to start modem...");

pinMode(MODEM_PWRKEY, OUTPUT);

digitalWrite(MODEM_PWRKEY, HIGH);

delay(300);

digitalWrite(MODEM_PWRKEY, LOW);

}

}

// Restart the modem if needed

SerialMon.println("Initializing modem...");

if (!modem.restart()) {

Serial.println("Failed to restart modem, attempting to continue without restarting");

}

}

// Function to perform an HTTP POST request with collected door numbers

void postReq(String doors) {

Serial.print("doorNumData:");

Serial.println(doors);

// Connect to the server

if (client.connect(server, 80)) {

// Send HTTP POST request

client.println("POST " + String(path) + " HTTP/1.1");

client.println("Host: " + String(server));

client.println("Connection: close");

client.println("Cache-Control: no-cache");

client.println();

// Send the collected data

client.println(doors);

// Read and print the server response

while (client.connected()) {

if (client.available()) {

char c = client.read();

Serial.print(c);

}

}

client.stop();  // Close the connection

} else {

Serial.println("connection failed");

}

}

// Function to blink an LED

void blinkLED(int pin) {

digitalWrite(pin, LOW);

delay(1500);

digitalWrite(pin, HIGH);

delay(1500);

}

Web Map :

Note: Place the web Files in same Folder and Paste the content as it is…

- CSV File

Door Num, GPS Lang, GPS Lati, Status

door.csv

DOOR1,34.052417,-118.365528,0

DOOR2,34.052171,-118.365892,0

DOOR3,34.052237,-118.365821,0

DOOR4,34.052335,-118.365809,0

DOOR5,34.052362,-118.365796,0

DOOR6,34.052396,-118.365786,0

- csvmap.php

<?php

// Avoid caching for the file

header('Cache-Control: no-store, no-cache, max-age=0, must-revalidate, proxy-revalidate');

header('Cache-Control: post-check=0, pre-check=0', false);

header('Pragma: no-cache');

// Read the content of the 'door.csv' file into an array ($file), each line as an element

$file = file('door.csv');

// Extract the door numbers from the URL parameter and convert it to an array

$doorNumbers = explode(",", $_GET['doorNumbers']);

// Loop through each line in the CSV file

foreach ($file as $index => $line) {

$doorDetails = explode(',', $line); // Split the line into an array by comma

$doorNumber = intval(trim($doorDetails[0], 'DOOR')); // Extract the door number and convert it to an integer

// Check if the door number is in the list to be updated

if (in_array($doorNumber, $doorNumbers)) {

$doorDetails[3] = '1'; // Modify the status to 1

} else {

$doorDetails[3] = '0'; // Set status to 0 for doors not in the list

}

$file[$index] = implode(',', $doorDetails). "\n"; // Update the line in the file

}

// Save the modified data back to the file

file_put_contents('door.csv', implode('', $file));

echo 'CSV file updated successfully!';

?>

- Map7.html

<!DOCTYPE html>

<html>

<head>

<title>OpenStreetMap with Markers from CSV</title>

<!-- Include Leaflet CSS and JavaScript -->

<link rel="stylesheet" href="https://unpkg.com/leaflet@1.7.1/dist/leaflet.css" />

<script src="https://unpkg.com/leaflet@1.7.1/dist/leaflet.js"></script>

<style>

/* Set the size of the map */

#map {

height: 1500px;

width: 100%;

}

</style>

</head>

<body>

<!-- The div element for the map -->

<div id="map"></div>

<script>

var map = L.map('map').setView([34.052396, -118.365786], 17); // Create a Leaflet map centered at specific coordinates

var markers = {}; // To store markers

// Add OpenStreetMap tile layer to the map

L.tileLayer('https://{s}.tile.openstreetmap.org/{z}/{x}/{y}.png', {

attribution: 'Map data &copy; <a href="https://www.openstreetmap.org/">OpenStreetMap</a> contributors',

maxZoom: 23

}).addTo(map);

// Fetch data from the PHP script that serves the CSV file

fetch('csvmap.php'); // Replace 'csvmap.php' with your PHP script URL

function fetchAndUpdateMarkers() {

// Fetch the CSV file with a timestamp to avoid caching

fetch('door.csv?' + new Date().getTime())

.then(response => response.text())

.then(data => {

var lines = data.split('\n'); // Split the CSV data into lines

lines.forEach(line => {

var parts = line.split(',');

var doorName = parts[0].trim(); // Extract door name

var coordinates = parts.slice(1).join(',').trim().replace(/[\[\]"']/g, ''); // Extract coordinates and status

var [latitude, longitude, status] = coordinates.split(',').map(coord => parseFloat(coord.trim()));

// Check the status to decide whether to show or remove the marker

if (status === 1) {

if (markers[doorName]) {

markers[doorName].setLatLng([latitude, longitude]); // Update marker position if exists

} else {

var marker = L.marker([latitude, longitude]).addTo(map); // Create a new marker if not exists

markers[doorName] = marker; // Store marker reference

marker.on('click', function() {

// Open Google Maps with marker's location on click

var url = 'https://maps.google.com/?q=' + this.getLatLng().lat + ',' + this.getLatLng().lng;

window.open(url, '_blank');

});

}

} else {

// Remove marker if status is not '1'

if (markers[doorName]) {

map.removeLayer(markers[doorName]);

delete markers[doorName];

}

}

});

})

.catch(error => {

console.error('Error fetching the CSV file:', error);

});

}

// Initially fetch and update markers

fetchAndUpdateMarkers();

// Update markers every 5 seconds

setInterval(fetchAndUpdateMarkers, 5000);

</script>

</body>

</html>

WebApp Working:

The Web Map will show mark according to the csv file if the status is 1 it will mark location on map if 0 then it will disappear from map.

Map Checks csv every 10 sec.

Esp updates csv every 30 sec.