Main idea of this project: Smart  IOT Scales

Main purpose: Save items to memory of the device to count their qty.

Modules: main board ESP32-2432s028R, HX711 (load cell amplifier)

In the future main boards might have different resolutions and/or capacitive touch screen, please take it into account.

I could consider using just esp32 and some SPI touchscreen. Because in the future this device might have its own PCB.

Requirements:
LVGL library, JSON, MQTT

Main functions:

1) Measuring weight

2) Counting items ( total weight / Known weight of saved item OR quick count ( total weight / preset qty (from input))

3) Tare

4) Wifi connectivity via touch screen and on-screen keyboard ( select wifi from the list of available networks and type password )

5) Saving items to memory ( add, edit, delete functions with screen )

6) Synchronizing items with remote server ( json via MQTT ) 
	Breakdown:
	Deviceis sending a message to MQTT server with serial number as a header.
	Every 1s ( constant in the code, so i could change it later) that its online.
	PS: Server side is not you business
	After server send sync request, device( scale ) sending json with data of saved items,
	After the server receives json, it will make some changes, add and/or delete items to json, and send it back to the device and device should save it to memory

7) Serial number generation ( MD5 Hash of mac address)

8) Unit selection ( make it easy to modify in the code )

9) Calibration of scales

10) firmware update via sd card ( but it will be cool to make update via wifi, not sure if its possible)

Main screen description:
1) Time automatically from internet

2) Battery status ( voltage reading for Li-Ion battery ) and wifi strength status
3) Small IMG of item ( getting from internet from saved URL ), showing only if link exist in the data. Name of the item and its weight

4) notifications ( scrolling row from right to left to read full message. Repeat 2 times)

5) Weight or counted QTY of items placed on the scales

6) Tare ( basic scales tare function )
7) Units selection ( drop up list, select from various units ( PCS ,kg, g, ml, l lb, etc.)

Note: When PCS selected, it should open full screen with list of saved items
8) Enter to the menu

9) The power button ( device goes into a deep sleep(for minimum power consumption). If possible to make double tap to wake up, would be cool)

10) Blank for now

Menu:
1) Items list:

List of saved items ( check attached picture )
With scrolling, add, back and sync buttons.
2) Calibration

3) settings:
	1) Wifi connection
	2) Device info ( serial number, firmware version, IP, MAC )
	3) Language ( for now just english )

4) auto power off timeout ( list of hardcoded minutes)

5) swipe scroll off/on

JSON example
{serialnumber: 14124123;

Message: {type: sync|status|etc;

Data: {some data};

}

Item data example:
{name:some name;
Img: url_to_img;

Weight: weight_of_items;

Qty: qty_of_items;}

Or make JSON better.

Why qty in items? Because there could be a situation when scale sensor is not sensitive and you need to measure 50 items and save their weigh as “a pack”